CPE301 – FALL 2019

# Midterm 1

Student Name: Alira Coffman

Student #: 5003236350

Student Email: coffma2@unlv.nevada.edu

Primary Github address: https://github.com/Alira-Coffman/submission-repo/

Directory: https://github.com/Alira-Coffman/submission-repo/tree/master/Midterm

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328p

Arduino Shield

Used the APC220(pin out below)

ESP8266:

https://i1.wp.com/henrysbench.capnfatz.com/wp-content/uploads/2016/12/ESP8266-PINOUT.png

LM35 temperature sensor:

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL

#include <avr/io.h>

#include <stdlib.h>

#include <avr/interrupt.h>

#include <stdio.h>

#include <util/delay.h>

#include <math.h>

#define BAUDRATE 115200

#define BAUD_PRESCALLER (int)round(((((double)F_CPU / ((double)BAUDRATE * 8.0))) - 1.0))

void USART_init(void);

void USART_transmit(unsigned char data);

void USART_putstring(char* StringPtr);

void ADC_init (void); //function to initialize ADC

void ESP_init();

void UART_sendString(volatile unsigned char AT[]);

float readTemperatue();

//VARIABLES

volatile float myTemp; //holds temp

char * tempString[10];

volatile unsigned char AT[];

void UART_sendString(volatile unsigned char AT[])

{

volatile unsigned char len = 0;

volatile unsigned char i;

while(AT[len] != 0)

{

len++;

}

for(i = 0x00; i < len; i++){

// Wait for the transmitter to finish

while(!(UCSR0A & (1<<UDRE0)));

UDR0 = AT[i];

}

}

int main(void)

{

USART_init(); //initialize USART

ADC_init (); //initialize ADC

ESP_init();

TCCR0A = 0x00; //set normal operation

TCCR0B |= (1 << CS02); //set prescalar to 256

TCNT0 = 6;

TIMSK0 |= (1 << TOIE0);

sei(); //set interrupt

while (1)

{

}

}

/*INITALIZE FUNCTIONS*/

void USART_init(void)

{

UCSR0A = (1 << U2X0);

UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8);

UBRR0L = (uint8_t)(BAUD_PRESCALLER);

UCSR0B = (1 << RXEN0) | (1 << TXEN0);

UCSR0C = (3 << UCSZ00);

}

void ESP_init()

{

UART_sendString("AT\r\n");

USART_putstring("AT+CWMODE=1\r\n");

_delay_ms(20);

USART_putstring("AT+CWJAP=\"Alira\",\"Password\"\r\n");

_delay_ms(20);

// Enable connection

UART_sendString("AT+CIPMUX=0\r\n");

_delay_ms(50);

}

void ADC_init (void)

{

ADMUX = (0<<REFS1)|

(1<<REFS0)|

(0<<ADLAR)|

(1<<MUX2)|

(0<<MUX1)|	// ADC5 PC4

(0<<MUX0);

ADCSRA = (1<<ADEN)|

(0<<ADSC)|

(0<<ADATE)|

(0<<ADIF)|

(0<<ADIE)|

(0<<ADPS2)|	// ADC Prescaler

(1<<ADPS1)|

(1<<ADPS0);

}

/*END OF INITS*/

void USART_transmit(unsigned char data)

{

while(!(UCSR0A & (1 << UDRE0)));

UDR0 = data;

}

float readTemperatue()

{

ADCSRA |= (1<<ADSC);

if(ADCSRA & (1<<ADSC));

myTemp = ADC;

_delay_ms(50);

}

void USART_putstring(char *StringPtr)

{

while ((*StringPtr != '\0')){

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = *StringPtr;

StringPtr++;

}

}

ISR(TIMER0_COMPA_vect)

{

_delay_ms(10);

UART_sendString("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n");

_delay_ms(20);

UART_sendString("AT+CIPSEND=45\r\n");

_delay_ms(20);

//     // Send temperature

UART_sendString("GET /update?key=IF6XQMA7HD4SUDOL&field1=");

readTemperatue();

snprintf(tempString, sizeof(tempString), "%f\r\n", myTemp);

UART_sendString(tempString);

UART_sendString("\r\n\r\n");

}

- SCHEMATICS

3.3v

ATMEGA

Arduino shield

gnd

WIFI

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- Video links

https://www.youtube.com/watch?v=AM_zD48u9p0&feature=youtu.be

- GITHUB LINK OF THIS DA

https://github.com/Alira-Coffman/submission-repo/tree/master/Midterm

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.